# CO₂ Water-Side Mass-Transfer Coefficients from Kanwisher (1963)

Kanwisher (1963) reported equivalent diffusion-film thicknesses for carbon dioxide (CO₂) of 127 µm under weak stirring and 47 µm under strong stirring (Table 1, p. 199). These are experimentally derived equivalent film thicknesses, not directly tabulated mass-transfer coefficients.

Using the stagnant-film relationship kL = DCO₂,w / δ, with the assumed CO₂ diffusivity in water DCO₂,w = 1.67606 × 10⁻⁵ cm²/s, the corresponding water-side transfer coefficients can be estimated as follows:

| Condition | Film thickness, δ | Calculated kL (m/s) |
| --- | --- | --- |
| Weak stirring | 127 µm (0.0127 cm) | 1.32 × 10⁻⁵ |
| Strong stirring | 47 µm (0.0047 cm) | 3.57 × 10⁻⁵ |

These coefficients are calculated from Kanwisher’s reported film thicknesses using the stated diffusivity; they are not numerical coefficients reported by Kanwisher. Neither the value 4.1 × 10⁻² m/s attributed to Kanwisher in a later publication nor its units could be verified in the original 1963 paper. The estimates should be treated as illustrative transfer conditions, not as a calibration for a 60-rpm orbital shaker.

## Stagnant-water sensitivity scenarios

For a non-stirred water surface, hypothetical film thicknesses of 200–1,000 µm were evaluated using direct fixed-film diffusion: kL,PCB4 = DPCB4,w / δ. With DPCB4,w calculated by molecular-weight scaling (approximately 8.10 × 10⁻⁶ cm²/s), Kaw = 10⁻¹·⁸⁷¹⁵⁵, and a provisional air-side reference coefficient of 0.003 m/s, the overall water-referenced coefficient was calculated from 1/ko = 1/kL,PCB4 + 1/(Kaw kG).

| Film thickness (µm) | Water-side kL (m/s) | Overall ko (cm/day) |
| --- | --- | --- |
| 127 | 6.37e-06 | 40.88 |
| 200 | 4.05e-06 | 28.65 |
| 500 | 1.62e-06 | 12.86 |
| 1000 | 8.10e-07 | 6.70 |

The 200–1,000 µm thicknesses are illustrative sensitivity assumptions, not values measured by Kanwisher or in the orbital-shaker experiment. The 127 µm row is a weak-stirring reference. The air-side coefficient is also provisional; these estimates should not be treated as calibrated values for a stagnant flask.

Reference: Kanwisher, J. (1963). On the exchange of gases between the atmosphere and the sea. Deep-Sea Research, 10, 195–207 (Table 1, p. 199).